- SDL Diagrams for the Packet Layer Protocol

The correct way to model the operation of the Packet Layer is to split it into two parts, the Restart Layer and the Packet Layer itself. The reason for this split is that there is an element of the Packet Layer (as defined in the protocol specification) that is independent of the operation of any one specific Packet Layer connection. Within the X.25 specification the committee attempt to model it within the confines of the existing Packet Layer but this leads to considerably more complex state diagrams than is necessary. The relationships between the various layers is best seen in the following diagram:

Packet Layer

Restart Layer

LAPB

As can be seen there is a one-to-one relationship between the “Restart Layer” and LAPB, but it is not part of the LAPB, and it would be a mistake to analyse or implement it in that context. For this reason the analysis of the Packet Layer is split into two parts.

4.3.1 The Restart Layer

The Restart Layer is relatively simple. The state diagram for this layer consists of four states:

Awaiting

Restart

2

Awaiting

Connection

1

Idle

0

Link

Active

3

Since this layer must interface to LAPB, it must send and receive the same messages as that understood by the LAPB state machine, for a full list of these messages reference should be made to the LAPB narrative in section 4.2. In addition this layer must interface to the Packet Layer and this requires an extra set of messages:

From the Packet Layer to the Restart Layer:

| Message Name | Meaning |
| --- | --- |
| RL Restart Request | Request that the Restart procedure be started. |
| RL Data Request | Data to be transmitted to the remote system. |

From the Restart Layer to the Packet Layer:

| Message Name | Meaning |
| --- | --- |
| RL Restart Confirmation | Confirms that a Restart requested by an RL Restart Request has completed successfully. |
| RL Restart Indication | Indicates that a Restart has been initiated by the remote system. |
| RL Failure Indication | Indicates that a non recoverable error has occurred. |
| RL Data Indication | Data has been received from the remote system. |

In addition the Restart Layer makes use of an internal timer, T300 which controls the intervals to send the Restart Requests at. No other variables, timers or flags are required.

X.25 Restart Layer

Idle State - State 0

Idle

0

0

idle

all

other

primitives

3

link

active

RL

Restart

Indication

Restart

Confirmation

Restart

Request

0

idle

all

other

primitives

1

awaiting

connection

DL

Connect

Request

RL

Restart

Request

X.25 Restart Layer

Awaiting Connection State - State 1

1

Awaiting

Connection

1

awaiting

connection

all

other

primitives

start T300

2

awaiting

restart

Restart

Request

DL

Connect

Confirmation

0

idle

RL

Failure

Indication

DL

Disconnect

Indication

1

awaiting

connection

all

other

primitives

X.25 Restart Layer

Awaiting Restart State - State 2

Awaiting

Restart

2

2

awaiting

restart

all

other

primitives

stop T300

0

idle

RL

Failure

Indication

DL

Disconnect

Indication

RL

Restart

Confirmation

stop T300

3

link

active

Restart

Confirmation

Restart

Confirmation

RL

Restart

Confirmation

stop T300

3

link

active

Restart

Request

2

awaiting

restart

all

other

primitives

Restart

Request

start T300

2

awaiting

restart

timer

T300

expiry

X.25 Restart Layer

Link Active State - State 3

Link

Active

3

DL

Data

Request

3

link

active

RL

Data

Request

3

link

active

all

other

primitives

3

link

active

Restart

Confirmation

Restart

Confirmation

RL

Restart

Indication

3

link

active

Restart

Request

0

idle

RL

Failure

Indication

DL

Disconnect

Indication

3

link

active

RL

Data

Indication

DL

Data

Indication

3

link

active

all

other

primitives

4.3.2 The Packet Layer

The Packet Layer has many of the features of LAPB, it has a similar connection start-up and tear-down phases and the data transfer phase is similar in many ways too. For this reason the state diagram of the Packet Layer bears a resemblance to that used for LAPB:

Awaiting

Reset

4

Data

Transfer

3

Awaiting

Connection

1

Dis-connected

0

Awaiting

Disconnection

2

Unlike LAPB the Packet Layer does not use the same Reject processing since it should never be needed. There is an option within the Packet Layer to enable the use of Reject frames, however this is not modelled here since it would appear that it is a rarely used option. The logic of processing this packet type would be broadly similar to that found in the LAPB state machine analysed earlier.

Some elements of the protocol are not modelled here. For example, the Packet Layer supports a standard (modulo 8) and an extended (modulo 128) mode in the same way as LAPB does, but unlike LAPB the different modes do not alter the way that the protocol operates, nor the packet types transferred. Therefore the different modes are left as a detail of the implementation and not of this model.

By relying on the reliability of LAPB, the designers were able to remove some of the more complex parts of the protocol that are used to ensure error detection and correction operates correctly, instead the main theme of the Packet Layer which is flow control. It is assumed that the Restart Layer has correctly negotiated the restart with the remote system, and that all interactions with the lower layers are directed via the Restart Layer. Therefore the only new primitives that need to be defined are between the Packet Layer and the user. These are detailed below:

From the Packet Layer to the user:

| Message Name | Meaning |
| --- | --- |
| PL Connect Confirmation | Confirms that a connection requested by a PL Connect Request has been successfully completed. |
| PL Connect Indication | Indicates that a connection initiated by the remote system has occurred. |
| PL Disconnect Confirmation | Confirms that a disconnection requested by a PL Disconnect Request has been successfully completed. |
| PL Disconnect Indication | Indicates that a disconnection initiated by the remote system has occurred. |
| PL Data Indication | Data that has been sent from the remote system has been received. |
| PL Interrupt Indication | Interrupt data that has been sent from the remote system has been received. |

From the user to the Packet Layer:

| Message Name | Meaning |
| --- | --- |
| PL Connect Request | Request a Packet Layer connection to the remote system. |
| PL Disconnect Request | Request a Packet Layer disconnection to the remote system. |
| PL Data Request | Data to be transmitted to the remote system |
| PL Interrupt Request | Interrupt data to be transmitted to the remote system, |
| Clear Own RX Busy | We can now accept data from the remote system. |
| Set Own RX Busy | We cannot accept any more data from the remote system for the time being. |

The Packet Layer requires a number of internal timers, variables and flags in order to operate correctly. The timers are prefixed with the number three in order to differentiate them from those in the Link Layer, these names are not those used by the X.25 specification. These are:

| Timer Name | Use |
| --- | --- |
| T301 | Time between sending repeated Call Requests. |
| T302 | Time between sending repeated Clear Requests. |
| T303 | Time between sending repeated Reset Requests. |
| T304 | Time to wait before sending an RR or an RNR in reply to data received. |

| Variable Name | Meaning |
| --- | --- |
| k | Maximum transmitted window size. |
| V(a) | Latest acknowledgement frame number transferred. |
| V(r) | Latest received frame number from the remote system. |
| V(s) | Latest transmitted frame number. |

| Flag Name | Meaning |
| --- | --- |
| Acknowledge Pending | Data has been received but has not been acknowledged by us yet. |
| Own Receiver Busy | The user is not willing to receive data at the moment. |
| Peer Receiver Busy | The remote system is not willing to receive data at the moment. |
| Interrupt Pending | Interrupt data that has been transmitted has not been acknowledged by the remote system. |

With this information the following SDL diagrams will make sense.

X.25 Packet Layer

Disconnected State - State 0

Discon-

nected

0

0

discon-

nected

PL

Disconnect

Confirmation

PL

Disconnect

Request

0

discon-

nected

all

other

primitives

start T302

2

awaiting

disconnection

Clear

Request

able to

establish?

V(s) ¬ 0

V(r) ¬ 0

V(a) ¬ 0

3

data

transfer

PL

Connect

Indication

clear

exception

conditions

Call

Confirmation

no

yes

Call

Request

Clear

Confirmation

0

discon-

nected

Clear

Request

0

discon-

nected

all

other

primitives

1

awaiting

connection

RL

Restart

Request

PL

Connect

Request

X.25 Packet Layer

Awaiting Connection State - State 1

Awaiting

Connection

1

1

awaiting

connection

all

other

primitives

Call

Confirmation

V(s) ¬ 0

V(r) ¬ 0

V(a) ¬ 0

clear

exception

conditions

stop T301

3

data

transfer

PL

Connect

Confirmation

Call

Request

Call

Confirmation

RL

Failure

Indication

Clear

Confirmation

PL

Disconnect

Indication

0

discon-

nected

stop T301

Clear

Request

1

awaiting

connection

all

other

primitives

RL

Restart

Indication

RL

Restart

Confirmation

start T301

1

awaiting

connection

timer

T301

expiry

Call

Request

X.25 Packet Layer

Awaiting Disconnection State - State 2

Awaiting

Dis-

connection

2

2

awaiting

disconnection

all

other

primitives

2

awaiting

disconnection

all

other

primitives

RL

Restart

Indication

RL

Failure

Indication

Clear

Confirmation

PL

Disconnect

Confirmation

stop T302

Clear

Confirmation

0

discon-

nected

Clear

Request

start T302

2

awaiting

disconnection

timer

T302

expiry

Clear

Request

X.25 Packet Layer

Data Transfer State - State 3  (1 of 3)

Data

Transfer

3

continued on next page

3

data

transfer

all

other

primitives

no

ack

pending?

enquiry

response

3

data

transfer

clear

Acknowledge

Pending

timer

T304

expiry

yes

start T303

stop T304

Reset

Request

4

awaiting

reset

all

other

primitives

3

data

transfer

Push I

frame on

queue

PL

Data

Request

3

data

transfer

Push I

frame on

int queue

PL

Interrupt

Request

stop T304

clear

Acknowledge

Pending

V(s) ¬ V(s) + 1

I

yes

yes

N(s) ¬ V(s)

N(r) ¬ V(r)

V(s) = V(a) + k

Peer

RX Busy ?

I frame

pops off

queue

no

no

3

data

transfer

Push I

frame on

queue

V(s) ¬ 0

V(r) ¬ 0

V(a) ¬ 0

3

data

transfer

stop T304

Reset

Confirmation

clear

exception

conditions

Reset

Request

X.25 Packet Layer

Data Transfer State - State 3  (2 of 3)

Data

Transfer

3

continued on next page

continued from previous page

RR

RNR

clear

Acknowledge

Pending

3

data

transfer

set

Own RX

Busy

set own

receiver

busy

Own RX

Busy ?

yes

no

clear

Acknowledge

Pending

clear

Own RX

Busy

clear own

receiver

busy

Own RX

Busy ?

yes

no

RL

Failure

Indication

RL

Restart

Indication

Clear

Request

PL

Disconnect

Indication

stop T304

Clear

Confirmation

discard

I frame

queue

0

discon-

nected

start T302

stop T304

Clear

Request

discard

I frame

queue

2

awaiting

disconnection

PL

Disconnect

Request

start T303

stop T304

Reset

Request

4

awaiting

reset

check

I frames

ack’d

3

data

transfer

set

Peer Receiver

Busy

RR

V(a) <=

N(r) <= V(s)

yes

no

clear

Peer Receiver

Busy

RNR

X.25 Packet Layer

Data Transfer State - State 3  (3 of 3)

Data

Transfer

3

continued from previous page

PL

Interrupt

Indication

3

data

transfer

Interrupt

Confirmation

Interrupt

clear

Interrupt

Pending

3

data

transfer

Interrupt

Confirmation

set

Interrupt

Pending

interrupt

yes

Interrupt

Pending ?

I frame

pops off

int queue

no

3

data

transfer

Push I

frame on

int queue

start T303

stop T304

4

awaiting

reset

Reset

Request

4

awaiting

reset

start T303

stop T304

Reset

Request

no

N(r) ¬ V(r)

3

data

transfer

RNR

clear

Acknowledge

Pending

Ack

Pending ?

Peer

RX Busy ?

PL

Data

Indication

check

I frames

ack’d

I

V(a) <=

N(r) <= V(s)

no

V(r) ¬ V(r) + 1

V(a) ¬ N(r)

Own RX

Busy ?

N(s) = V(r)

?

no

yes

yes

yes

yes

no

yes

3

data

transfer

set

Acknowledge

Pending

start T304

no

X.25 Packet Layer

Awaiting Reset State - State 4

Awaiting

Reset

4

4

awaiting

reset

all

other

primitives

V(s) ¬ 0

V(r) ¬ 0

V(a) ¬ 0

clear

exception

conditions

stop T303

3

data

transfer

Reset

Confirmation

RL

Restart

Indication

RL

Failure

Indication

PL

Disconnect

Indication

stop T303

Clear

Confirmation

0

discon-

nected

Clear

Request

4

awaiting

reset

Push I

frame on

int queue

I frame

pops off

int queue

4

awaiting

reset

Push I

frame on

queue

I frame

pops off

queue

4

awaiting

reset

Push I

frame on

queue

PL

Data

Request

4

awaiting

reset

Push I

frame on

int queue

PL

Interrupt

Request

4

awaiting

reset

all

other

primitives

start T303

4

awaiting

reset

Reset

Request

timer

T303

expiry

X.25 Packet Layer

Subroutines

clear

acknowledge

pending

clear

own receiver

busy

clear

interrupt

pending

clear

peer receiver

busy

clear

exception

conditions

no

yes

N(r) = V(s)

?

N(r) = V(a)

?

V(a) ¬ N(r)

no

V(a) ¬ N(r)

check

I frames

acked

yes