AA4RE BBS and the G8BPQ Switch.

******************************************************************************

This section is for BPQ switches V3.23 to 3.59

******************************************************************************

From version 3.23, the switch supports the AA4RE BBS.  This has been

done by extending the PK232 host mode support, originally written to

support the G8UFQ BBS.  The system allows the first Virtual TNC port to

support up to 8 users.  The configuration I used is detailed below:

In BPQCFG.TXT:

TYPE=PK232/AA4RE

APPLFLAGS=6

In PARMS.BB, the recommended port settings are below

port_type       = G8BPQ_NODE

com             = 2                ; use the first number in TNCPORTLIST

speed           = 4800             ; not actually used

max_connect     = 8                ; Up to 8

max_channel     = 9

max_pending     = 1

broadcast_path  = MAIL             ; is ignored - set in config.txt file

******************************************************************************

This section is for BPQ switches V4.02 and higher

******************************************************************************

No ports are needed in the BPQCFG.TXT.  I leave one token port

for testing:

TNCPORT

COM=4

ENDPORT

In PARMS.BB, the recommended port settings are below

port_type       = G8BPQ-HOST-x     ; The x is the port number on the

; BPQ switch.  Each port on the BPQ switch

; can be defined if you want to control

; monitor colors or issue broadcasts.

*** AND/OR ***

port_type       = G8BPQ-HOST       ; This is a generic port type for ALL

; the BPQ ports.  No broadcasts of MAIL

; waiting beacon will be sent.

com             = 127              ; Same number as HOSTINTERRUPT in the

; BPQCFG.TXT file

speed           = 4800             ; not actually used

max_connect     = 8                ; Up to 30??  Try 7

max_channel     = 9                ; Up to 30 or so.. Try 9

max_pending     = 4                ; Number of packets set to switch before

;       BBS waits

broadcast_path  = MAIL             ;

Note:  The only commands you can issue to the G8BPQ-HOST port are:

(esc)C -- Connect      *** See note 5 below ***

(esc)D -- Disconnect

(esc)M -- Monitor command.  Follow with letters to indicate what is

to be monitored

I = I frames

U = UI frames

S = Supervisory frames (RR, RNR, SABM, DM, D, etc)

F = Frames with PID byte F0 (hex) -- Normal

C = Frames with PID byte CF (hex) -- Netrom

A = Frames with any PID byte

N = No monitor

Examples:

MIUSA -- Monitor everything

MN -- Monitor off

You can define both G8BPQ-HOST-x and G8BPQ-HOST types in the same

PARMS.BB if you need only partial control.  For example, you have 3

ports but only want control over port 2.

port_type       = G8BPQ-HOST-2       ; This would be port 2

port_type       = G8BPQ-HOST         ; This would be all other ports

Special notes:

1) All connects will appear on the first port defined.  This

port will control MAXPAC, language, remote sysop, etc.

2) The port letter used will NOT determine which radio port will

be used.  You must use the proper switch sequence to get out

the proper port.  Thus a terminal task (T A5) will not ensure

that your packets will leave the radio port that is associated

with "A".

3) Each port will go thru its own forwarding cycle.  If you have

two ports, A and B, each will go thru their forwarding cycle

as per the port parameters.

4) Mail beacons will only occur for ports defined with the explicit

port_type.

5) The initial connect will determine which call sign is used for

outgoing connects.  Normally, the BBS call sign will be used.  You can

use the NODE call sign instead by specifying it on the outbound connect.

In terminal mode, use

(esc)C USENODECALL

In the path file, use

C USENODECALL

******************************************************************************

This section is for all BPQ switches

******************************************************************************

In PATHS.BB

PATH GB7DAD A GB7DAD FORCE

T 0000 2359

C SWITCH                                ; note initial C SWITCH

SC LRG7

R* to *

SC G4NAD-1

R* to *

All virtual TNCs continue to run as normal.  If you are running under

Desqview, you can still run PAC2 (or even MBL/RLI!) in other windows.

However, BB needs a LOT of store - you need to use all the tricks

described in the DV manual to get a big enough window to run it

effectively.