INTRODUCTION

Thanks for purchasing Memory Master for the Icom IC-706. I feel sure that

you will be pleased with the ease of operation  and the versatility  that

has gone into the production of this software. Programming the memory

channels from the front panel of a 706 can be a very time consuming and

frustrating operation. Memory Master allows you to make changes to the 706's

memory channels with ease. The program automatically sets up split

frequencies for repeaters by comparing the VFO A and VFO B frequencies that

you enter. If  the frequencies are different, the program automatically turns

on the SPLIT FUNCTION and stores the memory channel for repeater operation.

SETUP

Setup could not be easier. Just copy all the files to a directory of your

choice on your hard drive. The program can also be run from a floppy, but

it will be considerably slower.

*****VERY IMPORTANT*****

The IC-706 must be setup with  a CI-V ADDRESS of 48H  and a CI-V BAUD of 9600.

These functions can be checked by looking at the menu display. To access the

menu hold the LOCK button while pushing the POWER button. The radio will

come on in the menu mode. Check the following menu items:

Menu #                          Comment                          Value

21                              CI-V ADDRESS                     48H

22                              CI-V BAUD                        9600

23                              CI-V TRN                         on

You can look at each menu setting by pushing the UP or DOWN buttons.

Changes can be made to a menu item by turning the VFO KNOB until the

correct setting appears. Turn off the 706 by pushing the power switch to

save the settings.

OPERATION

At the dos prompt type MM.  The program will automatically read the

MEMORY.DUH and COM.INI files and display their contents on the screen.

The program will run with Windows 95 and Windows 3.1 and can be setup

with an appropriate icon.

Pressing the F1 key will advance the edit screen to each consecutive page.

The frequency data for each memory channel will be displayed as it was

stored in the MEMORY.DUH file.

To edit a memory channel, press the F1 key until the page with the desired

memory channel is displayed on the screen.

Press the F3 key and follow the prompts. If you enter anything incorrectly

the computer will indicate an incorrect response with a beep and allow you

to reenter the data. Enter the VFO A and VFO B frequencies in MHZ.

(Example 14.200)  If you enter 14.200 for VFO A and  14.200 for VFO B Memory

Master will automatically compare these frequencies and store 14.200 as a

simplex frequency. To program repeater frequencies enter the transmit

frequency into VFO A and the Receive frequency into VFO B. Memory Master

will compare these two frequencies and automatically store them as split

frequencies. (For Example: to store the 146.94 - repeater...you would enter

146.340 in VFO A and 146.940 into VFO B) As you edit memory channels they

will be highlighted on the edit screen with green print on a red field and

sent to the Memory.duh file.

Once  you have made all of the desired  frequency changes on the edit screen

you will be ready to program the IC-706. Check your com port setting !

Press F8 to toggle between COM 1 and COM 2.  Press F6 and sit back and

watch as your frequencies are programmed into the IC-706. It only takes

about 2 minutes on most computers. Memory Master will start programming  at

memory channel 1 and continue to program all channels through 99.

Memory Master will end with a beep and return back to the edit screen.

DEMO VERSION

The DEMO version of Memory Master is completely identical to the REGISTERED

version except it will only program the first six memory channels on the

IC-706. Please use the form provided in the register.doc file for

registration.

Thanks again for purchasing MEMORY MASTER. If you have any questions or

suggestions, please feel free to drop me a line. Ideas are welcomed !!!

I feel sure that this program will evolve as your suggestions come in and

I will repay your kindness by providing upgrades to registered users at a

VERY reasonable price.

Memory Master Software

Keith Thomas N5DUH

135 Kayla St.

Shreveport La. 71105

KEITH-N5DUH@worldnet.att.net