$OpenBSD: z8530.doc,v 1.2 1996/10/31 01:01:41 niklas Exp $

$NetBSD: z8530.doc,v 1.1 1996/01/24 01:07:21 gwr Exp $

Here are the results of my reorganization work on the zs driver.

Everything compiles and appears to work.

Enjoy!

Gordon Ross

----------------------------------------------------------------

This design has a "zs controller" named "zsc" with three

flavors of child drivers that can be attached above it.

The three child drivers are:

zstty:	normal RS-232 line

kbd:	sun keyboard

ms:	sun mouse

Note that the machine-dependent parts are:

arch/sun3/include/z8530var.h

arch/sun3/dev/zs.c

(replicate those for new systems)

The Sun hardware dependent parts (Sun3 and SPARC) are:

kd*	sun keyboard/display console stuff

event*	sun "firm event" stuff

----------------------------------------------------------------

Here is how the autoconfig looks on a sun3:

zsc0 at obio0 addr 0x0 level 6 softpri 3

kbd0 at zsc0 channel 0

ms0 at zsc0 channel 1

zsc1 at obio0 addr 0x20000 level 6 softpri 3

zstty0 at zsc1 channel 0 (console)

zstty1 at zsc1 channel 1

----------------------------------------------------------------

Config lines:

In any config file (i.e. GENERIC, NEW_ZS, ...)

Note {kbd,ms} are sun-specific.

#

# New console (zs) stuff

#

zsc0 at obio? addr ?

zsc1 at obio? addr ?

kbd0 at zsc0 channel 0

ms0  at zsc0 channel 1

zstty* at zsc? channel ?

----------------------------------------------------------------

In conf/files.sun3

#

# Console (zs) related stuff

#

device zsc at obio {channel = -1}

file arch/sun3/dev/zs.c 		zsc needs-flag

file dev/ic/z8530sc.c			zsc

device zstty at zsc: tty

file dev/ic/z8530tty.c			zstty needs-flag

define zsevent

file dev/sun/event.c			zsevent

device kbd at zsc: zsevent

file dev/sun/kbd.c			kbd needs-flag

file dev/sun/kbd_tables.c 		kbd

file arch/sun3/dev/kd.c 		kbd

device ms at zsc: zsevent

file dev/sun/ms.c			ms needs-flag